Fringe Projection Hardware & Geometry Summary

FLIR Blackfly + Edmund Optics Telecentric + Pico Genie Setup

For the Three.js virtual simulation collaborator

# 1. Hardware Inventory

Confirmed components currently in the lab:

| Component | Model / Spec | Role |
| --- | --- | --- |
| Camera | FLIR Blackfly S BFS-U3-13Y3M-C | 1280×1024 mono USB3, global shutter, 4.8 µm pitch |
| Camera lens | Edmund Optics #58-259 (TECHSPEC GoldTL) | 0.09× telecentric, WD 132–182 mm, < 0.2° telecentricity |
| Projector (current) | Pico Genie Impact 2.0 Plus Elite | DLP 854×480, 1.2:1 throw, NON-telecentric, placeholder |
| Projector (planned) | TBD upgraded unit | Telecentric status unknown — to be confirmed |

# 2. Camera

FLIR Blackfly S — identified from sticker on body during physical inspection. Industrial monochrome USB3 Vision machine vision camera, suitable for fringe projection metrology.

| Parameter | Value |
| --- | --- |
| Manufacturer | Teledyne FLIR (sold by Edmund Optics, stock #36-451) |
| Model | Blackfly S BFS-U3-13Y3M-C |
| Serial number | 26048170 |
| Sensor | ON Semi PYTHON 1300, 1/2", global shutter |
| Resolution | 1280 × 1024 (1.3 MP) |
| Pixel size | 4.8 × 4.8 µm |
| Sensor active area | 6.14 × 4.92 mm |
| Frame rate (max) | 170 fps |
| Color | Monochrome |
| Interface | USB3 Vision |
| Mount | C-mount |
| Body size | 29 × 29 × 30 mm, 36 g |
| SDK | Spinnaker (Teledyne FLIR) with PySpin Python bindings |

# 3. Camera Lens (Telecentric)

Edmund Optics #58-259 — confirmed via the 0.09× and 58259 markings on the lens body.

| Parameter | Value |
| --- | --- |
| Manufacturer | Edmund Optics |
| Stock number | #58-259 |
| Series | TECHSPEC GoldTL Focusable Telecentric |
| Magnification (modern) | 0.09× (sensor size / object size) |
| Magnification (chapter convention) | ≈ 11.1× (1 / 0.09) |
| Working distance | 132 – 182 mm (focusable) |
| Telecentricity | < 0.2° |
| Sensor format | 1/2" (matches FLIR camera) |
| Status | Telecentric — viewing-arm perspective bias is HARDWARE-eliminated |

Critical: the telecentric viewing lens means the camera arm contributes essentially zero perspective bias. Any measurable bias in the system comes from the projection arm only.

# 4. Derived Optical Parameters

Quantities computed from the camera + lens specs:

| Quantity | Value | Derivation |
| --- | --- | --- |
| Field of view (horizontal) | ≈ 68 mm | Sensor 6.14 mm / mag 0.09 |
| Field of view (vertical) | ≈ 55 mm | Sensor 4.92 mm / mag 0.09 |
| Object-space pixel pitch | ≈ 53 µm | Pixel 4.8 µm / mag 0.09 |
| Imaging-arm M (modern) | 0.09× | From lens datasheet |
| Imaging-arm M (chapter) | ≈ 11.1 | 1 / 0.09 — for chapter equations |

Magnification convention warning: the chapter (Samara thesis) defines M as object-distance / sensor-distance, so its M values are >1. The Edmund Optics datasheet uses the modern machine-vision convention (sensor / object). When plugging into chapter equations, use the chapter-convention value (≈ 11.1) — using 0.09 directly will give answers off by a factor of ~123×.

# 5. Projector Coordinate System

All projector measurements are expressed in the projector's body frame:

- Origin: front-bottom-LEFT corner of the projector cube (lens-facing side)

- +X axis: along the front-bottom edge, rightward when facing the projector

- +Y axis: from the front face going INTO the body (away from the wall)

- +Z axis: vertical, pointing upward

The projection wall sits at Y = −D where D is the throw distance.

# 6. Projector Body and Lens Position

| Parameter | Value | Units | Notes |
| --- | --- | --- | --- |
| Cube width (X) | 55 | mm | Per spec sheet |
| Cube depth (Y) | 55 | mm | Per spec sheet |
| Cube height (Z) | 55 | mm | Per spec sheet |
| X_lens | 21 | mm | From front-bottom-left corner, rightward |
| Z_lens | 45 | mm | From front-bottom edge, upward |
| Y_lens | ≈ 1–2 | mm | Slight recess inside front face (estimated) |
| Lens diameter | TBD | mm | Not yet measured |

Key takeaway: the lens center is offset to the LEFT of the cube center (21 mm vs. 27.5 mm) and toward the TOP of the front face (45 mm of 55 mm total height). Normal layout for a pico projector.

# 7. Throw Geometry

Measured at a throw distance of 30 cm:

| Parameter | Value | Notes |
| --- | --- | --- |
| Throw distance D | 300 mm (30 cm) | Lens to wall, perpendicular |
| Image width (measured) | ≈ 261 mm | Direct measurement on wall |
| Image width (predicted) | 250 mm | From 1.2:1 throw ratio |
| Image height (measured) | 144 mm | Direct measurement on wall |
| Image height (predicted) | ≈ 141 mm | From 16:9 aspect ratio |
| Throw ratio (spec) | 1.2 : 1 | Width = D / 1.2 |
| Aspect ratio (spec) | 16 : 9 | Height = Width × 9/16 |

Sanity check: measured image dimensions agree with the throw-ratio prediction within ~4%, well within manual measurement noise.

# 8. Optical Lens Offset

Deviation between the lens mechanical axis and the chief ray to the image center.

| Offset axis | Value | Status |
| --- | --- | --- |
| Vertical offset (Δh) | 0.0 cm | CONFIRMED — well-measured |
| Vertical offset angle | 0° | CONFIRMED |
| Vertical offset percentage | 0% | CONFIRMED |
| Horizontal offset (Δw) | ≈ 6.6 cm | TENTATIVE — possible setup misalignment |
| Horizontal offset angle | ≈ 12.4° | TENTATIVE — see assumptions |
| Horizontal offset percentage | ≈ −25% | TENTATIVE — image shifted left |

Vertical offset (CONFIRMED): the projected image center sits at the same vertical height as the lens center. The image height is symmetric about the lens optical axis.

Horizontal offset (TENTATIVE): wall measurement suggests a ~6.6 cm shift (image center 12.4° to the left of the lens axis). However, this could also be explained by a small yaw misalignment of the hand-positioned projector. Could not be cleanly separated. To be re-measured once proper kinematic mounting is available.

# 9. Projected Image Corner Coordinates

Image corners on the wall plane (Y = −300 mm), in projector body frame:

| Corner | X (mm) | Y (mm) | Z (mm) |
| --- | --- | --- | --- |
| Bottom-Left | −176 | −300 | −27 |
| Bottom-Right | +85 | −300 | −27 |
| Top-Left | −176 | −300 | +119 |
| Top-Right | +85 | −300 | +119 |
| Image center | −45.5 | −300 | +46 |

## Parametric formula (general throw distance D)

Width on wall: W(D) = D / 1.2

Height on wall: H(D) = W × 9/16 ≈ 0.469 × D

Corner positions (assuming 0° horizontal AND vertical offset — see assumptions):

- X_left  = X_lens − W/2,   X_right = X_lens + W/2

- Z_bot   = Z_lens − H/2,   Z_top   = Z_lens + H/2

- Y_corner = −D for all four corners

Note: if horizontal offset turns out to be real (not setup artifact), X_left and X_right need an additional offset term.

# 10. Optical Configuration Summary

- Viewing arm: TELECENTRIC (Edmund Optics 0.09×). Camera-side perspective bias is hardware-eliminated.

- Projection arm: NON-TELECENTRIC (Pico Genie consumer DLP). Projection-side perspective bias must be corrected via inverse grating method.

- System type: HYBRID (asymmetric).

- Correction strategy: Inverse grating per Chapter 4 §4.3.1 — empirically calibrated, no system-parameter measurements required.

# 11. Assumptions and Caveats

## Assumed values

- Projector cube dimensions exactly 55 × 55 × 55 mm (per spec sheet — not yet directly verified).

- Throw ratio 1.2:1 and aspect ratio 16:9 (per spec sheet).

- Lens optical axis parallel to +Y direction in body frame (perpendicular to front face).

- Y_lens recess depth ≈ 1–2 mm; for simulation, use 1.5 mm. Negligible effect on geometry at throw distances ≥ 30 cm.

- Camera lens magnification 0.09× per Edmund Optics datasheet.

## Setup conditions during measurement

- Projector hand-positioned on a tissue box (no rigid mounting hardware available yet).

- Auto-keystone: confirmed OFF on the projector menu.

- 4-corner correction and manual keystone: assumed OFF (image was rectangular).

- Focus: adjusted manually until image edges sharp.

- Front projection mode selected.

## Items still TBD

- Lens diameter (camera + projector) — physical measurement.

- Cube dimensions verification with calipers.

- Re-measurement of horizontal offset on properly mounted projector.

- Verification of vertical offset at a second throw distance (e.g., 60 cm).

- Upgraded projector specs (when hardware arrives).

# 12. Recommendation for Initial Simulation

For the first iteration of the virtual simulation, use these simplifying assumptions:

- Lens position in body frame: (X = 21 mm, Y = 1.5 mm, Z = 45 mm).

- Lens optical axis: parallel to +Y.

- Vertical lens offset: 0° (well-supported by measurement).

- Horizontal lens offset: 0° (TENTATIVE — measurement suggests ~12° but may be misalignment artifact). Plan to revisit once real mounting hardware allows clean re-measurement.

- Throw ratio 1.2:1, aspect ratio 16:9.

- Camera magnification 0.09× (modern convention) or ≈ 11.1 (chapter convention).

This lets the simulation proceed without blocking on hardware. Once proper mounts arrive, redo the horizontal offset measurement and update accordingly. Vertical geometry should remain unchanged.